Електротехнички факултет у Београду

Катедра за рачунарску технику и информатику

Пројекат из предмета

Оперативни системи 1

|  | датум предаје: | Петровић Дарко |
| --- | --- | --- |
|  | 23. септембар 2005. | 03/119 |

Садржај

Увод	3

Изворни код	4

thread.h	4

semaphor.h	5

event.h	6

schedul.h	7

hwdepend.h	8

idle.h	9

kernel.h	10

kernelev.h	11

krnlsem.h	12

mytypes.h	13

pcb.h	14

queue.h	15

sleeping.h	16

event.cpp	17

hwdepend.cpp	18

idle.cpp	20

kernel.cpp	21

kernelev.cpp	23

krnlsem.cpp	24

main.cpp	25

pcb.cpp	26

queue.cpp	27

semaphor.cpp	28

sleeping.cpp	29

thread.cpp	30

Увод

Приликом реализације пројекта, усвојене су следеће претпоставке:

- пошто се може догодити да распоређивач (класа Scheduler) буде празан, претпостављено је да у том случају метода Scheduler::get() враћа празан показивач (NULL)

- подаци у оквиру корисничке нити (попут информација о току њеног извршавања) могу бити потребни и након што се заврши метода run(), па се меморија додељена нити не ослобађа имплицитно по њеном завршетку

Захтев да се дата заглавља не мењају није у потпуности испоштован. Наиме, у заглавље thread.h додата је декларација friend class Kernel, да би класа Kernel, која представља само језгро, могла да приступи операцији run(). Ово се могло постићи и без промене заглавља, коришћењем класе PCB за приступ заштићеној операцији, али коришћени начин је природнији и лепши, а интерфејс према кориснику остаје исти. Осим ове измене, у дефиницију саме класе Thread додате су и јавне методе sleep() и join(), описанe у наставку. Остала заглавља остала су неизмењена.

По угледу на интерфејс коришћен у програмском језику Java, додата је могућност ,,успављивања” нити методом sleep(), којој се као аргумент предаје време после чијег истека се нит буди, тј. враћа у трку за добијање процесора. За потребе ове методе, реализована је класа SleepingQueue, која садржи успаване нити. Њена статичка метода tick() позива се приликом прекида од системског тајмера и као резултат даје број нити које су у том позиву враћене у распоређивач, тј. ,,пробуђене”, што може бити од користи при даљем унапређивању језгра. Такође, по угледу на номенклатуру коју намеће Java, метода waitToComplete() добила је нови назив, join(). Наравно, и старо име задржано је у интерфејсу, због компатибилности са тест примерима.

Услед недостатка подршке преводиоца за руковање изузецима, реакције језгра на бројне грешке које могу настати приликом извршавања (неуспела алокација меморије, покушај вишеструког стартовања језгра итд.) углавном нису предвиђене. Наравно, евентуалне проблеме могуће је решити коришћењем других концепата, али један од циљева приликом унапређивања језгра јесте прилагодити га савременијем преводиоцу, после чега се грешке могу обрађивати на природнији начин.

Изворни код

//===========================================================================//

//   Project: Projekat iz Operativnih sistema 1

//   File:    thread.h

//   Date:    June 2005

//===========================================================================//

#ifndef _OS1_THREAD_H_

#define _OS1_THREAD_H_

//---------------------------------------------------------------------------//

// time in milliseconds

//---------------------------------------------------------------------------//

typedef float Time;

typedef unsigned long StackSize;

Time minTimeSlice();

const StackSize defaultStackSize = 4096;

const Time defaultTimeSlice = 100;

//---------------------------------------------------------------------------//

void dispatch ();

//---------------------------------------------------------------------------//

class PCB; // Kernel's implementation of a user's thread

//---------------------------------------------------------------------------//

class Thread {

friend class PCB;

friend class Kernel;

public:

void start();

void waitToComplete();

void join() { waitToComplete(); }  //just another name for waitToComplete()

void sleep(Time);                  //New functionality added!

virtual ~Thread();

protected:

Thread (StackSize stackSize=defaultStackSize, Time timeSlice=defaultTimeSlice);

virtual void run() {}

private:

PCB* myPCB;

};

#endif // _OS1_THREAD_H_

//===========================================================================//

//   Project: Projekat iz Operativnih sistema 1

//   File:    semaphor.h

//   Date:    June 2005

//===========================================================================//

#ifndef _OS1_SEMAPHOR_H_

#define _OS1_SEMAPHOR_H_

class KernelSem;

extern int semPreempt;

class Semaphore {

public:

Semaphore (int init=1);

~Semaphore ();

void wait  ();

void signal();

int val () const;  // Returns the current value of the semaphore

private:

KernelSem* myImpl;

};

#endif  // _OS1_SEMAPHOR_H_

//===========================================================================//

//   Project: Projekat iz Operativnih sistema 1

//   File:    event.h

//   Date:    June 2005

//===========================================================================//

#ifndef _OS1_EVENT_H_

#define _OS1_EVENT_H_

typedef unsigned int IVTNo;

typedef void interrupt (*InterruptHandler)(...);

class KernelEv;

class Event {

public:

Event (IVTNo, InterruptHandler);

~Event ();

void wait  ();

void signal();

void oldHandler ();

private:

KernelEv* myImpl;

};

//---------------------------------------------------------------------------//

#endif  // _OS1_EVENT_H_

//===========================================================================//

//   Project: Projekat iz Operativnih sistema 1

//   File:    schedul.h

//   Date:    June 2005

//===========================================================================//

#ifndef _OS1_SCHEDUL_H_

#define _OS1_SCHEDUL_H_

class PCB;

class Scheduler {

public:

static void put (PCB*);

static PCB* get ();

};

#endif // _OS1_SCHEDUL_H_

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          hwdepend.h

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Hardware dependant parts of the threading system.

//                  Currently designed for Intel 8086 compatibile CPU-s

//===========================================================================//

#ifndef _LOCK_H_

#define _LOCK_H_

#include <dos.h>

#include "mytypes.h"

///////////////////   Borland C++ 3.1 Compiler options   //////////////////////

#if defined(__TINY__) || defined(__SMALL__) || defined(__MEDIUM__) ||   \

defined(__COMPACT__) || defined(__HUGE__)

#error Large memory model needed!!!

#endif

///////////////////////////////////////////////////////////////////////////////

#define TIMER_IVTNo 0x08    //The number of the IVT entry for system timer ISR

#define TIMER_FREQ 55       //Delay between system timer interrupts

#define lock() asm cli; locksCnt++;

#define unlock() if(!(--locksCnt)) { asm sti; }

#define saveStack(stack)              \

asm {                             \

push locksCnt;                \

mov sseg, ss;                 \

mov sptr, sp                  \

};                                \

stack = (byte*)MK_FP(sseg,sptr);

#define restoreStack(stack)           \

sseg = FP_SEG((stack));           \

sptr = FP_OFF((stack));           \

asm {                             \

mov ss, sseg;                   \

mov sp, sptr;                   \

pop locksCnt                    \

}

//defined in cpp file

extern unsigned sseg, sptr;

extern int locksCnt;

byte* initStack(void (*body)(), StackSize size, byte* stack);

//setvect() and getvect() can be used instead of setISR()

InterruptHandler setISR(IVTNo, InterruptHandler);

#endif

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          idle.h

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Idle thread - used when scheduler runs out of ready threads

//===========================================================================//

#ifndef _IDLE_H_

#define _IDLE_H_

#include "thread.h"

#define IDLE_STACK_SIZE 128     //more then enough for the idle thread

class Idle: public Thread {

protected:

virtual void run();

private:

Idle();

friend class Kernel;        //only Kernel can create the idle thread

};

#endif

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          kernel.h

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   User interface of the threading system

//===========================================================================//

#ifndef _KERNEL_H_

#define _KERNEL_H_

#include "mytypes.h"

#include "pcb.h"

#include "semaphor.h"

class Kernel {

/////////////////////////  USER INTERFACE  ////////////////////////////////////

public:

static void setEnabled();         //switch the threading system on...

static void setDisabled();        //...or off

inline static unsigned getThreadCount(); //get the number of active threads

///////////////////////////////////////////////////////////////////////////////

private:

Kernel();  //there is no need to create objects of this class

static void interrupt timerISR();          //new ISR for the system timer

static InterruptHandler defaultTimerISR;   //pointer to the default timer ISR

static void wrapper();

static void wait();

static void signUp();

static void remove();

volatile static long timeRemaining;

volatile static int explicitDispatch, timeSharing;

static int enabled;

static PCB* mainPCB, *idlePCB;

static volatile PCB* running;

static unsigned threadCounter;

friend class Thread;

friend class PCB;

friend class KernelSem;

friend class KernelEv;

friend void dispatch();

};

#endif

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          kernelev.h

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Kernel event interface - used by Event class only

//===========================================================================//

#ifndef _KERNELEV_H_

#define _KERNELEV_H

#include "mytypes.h"

#include "queue.h"

class KernelEv {

KernelEv(IVTNo, InterruptHandler);

~KernelEv();

void wait();

void signal();

InterruptHandler oldISR;      //points to the old handler

IVTNo IVTnumber;

Queue* blocked;

friend class Event;

};

#endif

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          krnlsem.h

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Kernel semaphore interface - used by Semaphore only

//===========================================================================//

#ifndef _KRNLSEM_H_

#define _KRNLSEM_H_

class Queue;

class KernelSem {

KernelSem(int init);

~KernelSem();

void wait();

void signal();

int value;

Queue* blocked;

friend class Semaphore;

};

#endif

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          mytypes.h

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Typedefs used in the project

//===========================================================================//

#ifndef _MYTYPES_H_

#define _MYTYPES_H_

typedef float Time;

typedef unsigned long StackSize;

typedef char byte;

typedef unsigned int IVTNo;

typedef void interrupt (*InterruptHandler)(...);

#endif

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          pcb.h

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Kernel thread interface

//===========================================================================//

#ifndef _PCB_H_

#define _PCB_H_

#include "mytypes.h"

class Thread;

class Queue;

class PCB {

PCB(StackSize stackSize, Time timeSlice, Thread* thread);

~PCB();

enum State { NEW, READY, BLOCKED, SLEEPING, DONE };

State state;

byte* currStack, *forDealloc;    //forDealloc is used when the PCB

Time tslice, sleepTime;          //is being destroyed

Thread* myT;

Queue* blocked;

friend class Thread;

friend class Kernel;

friend class KernelSem;

friend class KernelEv;

friend class SleepingQueue;

};

#endif

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          queue.h

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   The simple waiting queue

//===========================================================================//

#ifndef _QUEUE_H_

#define _QUEUE_H_

class PCB;

class Elem {

public:

Elem(PCB* pcb);

PCB* info;

Elem* next;

};

class Queue {

/////////////////////////  USER INTERFACE  ////////////////////////////////////

public:

Queue();

~Queue();

void put(PCB* pcb);

PCB* get();

int size() { return cnt; }

///////////////////////////////////////////////////////////////////////////////

private:

Elem* first, *last;

int cnt;

friend class SleepingQueue;

};

#endif

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          sleeping.h

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Queue for sleeping threads

//===========================================================================//

#ifndef _SLEEPING_H_

#define _SLEEPING_H_

class PCB;

class Queue;

class SleepingQueue {

public:

static void put(PCB*);

static int tick();

private:

SleepingQueue();

static Queue sleeping;

};

#endif

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          event.cpp

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Event class implementation

//===========================================================================//

#include "event.h"

#include "hwdepend.h"

#include "kernelev.h"

Event::Event(IVTNo number, InterruptHandler ISR) {

lock();

myImpl = new KernelEv(number, ISR);

unlock();

}

Event::~Event() {

lock();

delete myImpl;

unlock();

}

void Event::wait() {

lock();

myImpl->wait();

unlock();

}

void Event::signal() {

lock();

myImpl->signal();

unlock();

}

void Event::oldHandler() { (*(myImpl->oldISR))(); }

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          hwdepend.cpp

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Hardware dependant parts of the threading system.

//                  Currently designed for Intel 8086 compatibile CPU-s

//===========================================================================//

#include "hwdepend.h"

unsigned sseg, sptr;    //used by saveStack and restoreStack macros in the header file

unsigned oldSS, oldSP;  //for temporary stack splitting in initStack function

unsigned tss, tsp, tcs, tip;    //inline assembly cannot operate class fields

int locksCnt = 0;   //nested locks counter

byte* initStack(void (*body)(), StackSize size, byte* stack) {

tss = FP_SEG((stack + size)); tsp = FP_OFF((stack + size));

tcs = FP_SEG(body);  tip = FP_OFF(body);

asm {

mov oldSS, ss

mov oldSP, sp

mov ss, tss

mov sp, tsp

pushf

pop ax

or ax, 200h

push ax

push tcs

push tip

push ax

push bx

push cx

push dx

push es

push ds

push si

push di

push bp

mov ax, 0     //push 0 - initial locksCount

push ax

mov tsp, sp

mov ss, oldSS

mov sp, oldSP

}

stack = (byte*)MK_FP(tss, tsp);

return stack;

}

InterruptHandler setISR(IVTNo no, InterruptHandler isr) {

lock();

unsigned oldRoutineSEG, oldRoutineOFF;

unsigned newRoutineSEG = FP_SEG(isr), newRoutineOFF = FP_OFF(isr);

InterruptHandler oldRoutine;

no*=4;

asm {

push es

push ax

push bx

mov ax, 0

mov es, ax

mov bx, no;

push word ptr es:bx

pop oldRoutineOFF

add bx, 2

push word ptr es:bx

pop oldRoutineSEG

push newRoutineSEG

pop word ptr es:bx

sub bx, 2

push newRoutineOFF

pop word ptr es:bx

pop bx

pop ax

pop es

}

oldRoutine = (InterruptHandler)(MK_FP(oldRoutineSEG, oldRoutineOFF));

unlock();

return oldRoutine;

}

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          idle.cpp

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Idle thread implementation

//===========================================================================//

#include "idle.h"

Idle::Idle(): Thread(IDLE_STACK_SIZE, minTimeSlice()) {}

void Idle::run() { for(;;); }  //The idle thread simply enters an infinite loop

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          kernel.cpp

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   The implementation of the threading system

//===========================================================================//

#include <_null.h>

#include "hwdepend.h"

#include "idle.h"

#include "kernel.h"

#include "pcb.h"

#include "queue.h"

#include "schedul.h"

#include "sleeping.h"

#include "thread.h"

/////////////////// Static fields must be initialized first ///////////////////

volatile long Kernel::timeRemaining = 0;

volatile int Kernel::explicitDispatch = 0;

volatile int Kernel::timeSharing = 1;

int Kernel::enabled = 0;

unsigned Kernel::threadCounter = 0;

PCB* Kernel::mainPCB = (new Thread(4096))->myPCB;

volatile PCB* Kernel::running = Kernel::mainPCB;

PCB* Kernel::idlePCB = (new Idle())->myPCB;

InterruptHandler Kernel::defaultTimerISR = NULL;

///////////////////////////////////////////////////////////////////////////////

void tick();    //provided by user

void Kernel::setEnabled() {

if(!enabled) {

enabled = 1;

mainPCB->state = PCB::READY;     //state was NEW initially

defaultTimerISR = setISR(TIMER_IVTNo, (InterruptHandler)timerISR);

}

}

void Kernel::setDisabled() {

if(enabled) {

wait();   //all threads must be done before default ISR can be restored

setISR(TIMER_IVTNo, defaultTimerISR);

enabled = 0;

}

}

void Kernel::wrapper() {

running->myT->run();

lock();

while(((PCB*)running)->blocked->size()) {        //empty the waiting queue

PCB* p = ((PCB*)running)->blocked->get();

p->state = PCB::READY;

Scheduler::put(p);

}

running->state = PCB::DONE;

Kernel::remove();

unlock();

dispatch();

}

void interrupt Kernel::timerISR() {

if(!explicitDispatch) {             //if an interrupt has occured

(*defaultTimerISR)();           //call the old ISR

timeRemaining -= TIMER_FREQ;    //decrement remaining time

SleepingQueue::tick();

tick();

}

//if time exceeded or thread called dispatch, switch the context

if((timeRemaining<=0 && timeSharing) || explicitDispatch) {

explicitDispatch = 0;

saveStack(running->currStack);    //save the old context...

//...and if the thread is still ready, put it back to the scheduler

if(running->state==PCB::READY) Scheduler::put((PCB*)running);

running = Scheduler::get();

//in the case of empty scheduler, switch to idle

if(running==NULL) running = idlePCB;

timeRemaining = running->tslice;

timeSharing = timeRemaining?1:0;

restoreStack(running->currStack);    //restore the new context

}

}

void Kernel::signUp() { threadCounter++; }

void Kernel::remove() { threadCounter--; }

inline unsigned Kernel::getThreadCount() { return threadCounter; }

void Kernel::wait() {

while(threadCounter!=0) dispatch();

}

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          kernelev.cpp

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Kernel implementation of a user's event

//===========================================================================//

#include "hwdepend.h"

#include "kernel.h"

#include "kernelev.h"

#include "schedul.h"

KernelEv::KernelEv(IVTNo number, InterruptHandler isr) {

lock();

IVTnumber = number;

blocked = new Queue();

oldISR = setISR(number, isr);

unlock();

}

KernelEv::~KernelEv() {

lock();

setISR(IVTnumber, oldISR);    //restore the old handler

delete blocked;

unlock();

}

void KernelEv::wait() {

lock();

Kernel::running->state = PCB::BLOCKED;

blocked->put((PCB*)Kernel::running);

dispatch();

unlock();

}

void KernelEv::signal() {

lock();

while(blocked->size()) {     //put all blocked threads back to the Scheduler

PCB* p = blocked->get();

p->state = PCB::READY;

Scheduler::put(p);

}

dispatch();

unlock();

}

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          krnlsem.cpp

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Kernel implementation of a user's semaphore

//===========================================================================//

#include "hwdepend.h"

#include "kernel.h"

#include "krnlsem.h"

#include "queue.h"

#include "schedul.h"

int semPreempt;

KernelSem::KernelSem(int init): value(init), blocked(new Queue()) {}

KernelSem::~KernelSem() { delete blocked; }

void KernelSem::wait() {

lock();

if(--value < 0) {

Kernel::running->state = PCB::BLOCKED;

blocked->put((PCB*)Kernel::running);

dispatch();

}

else if(semPreempt) dispatch();

unlock();

}

void KernelSem::signal() {

lock();

if(++value <= 0) {

PCB* p = blocked->get();

p->state = PCB::READY;

Scheduler::put(p);

}

if(semPreempt) dispatch();

unlock();

}

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          main.cpp

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Main function.

//                  Simply starts the kernel and then calls user's application.

//===========================================================================//

#include <iostream.h>

#include "kernel.h"

int userMain(int argc, char* argv[]);   //user's application

int main(int argc, char* argv[]) {

Kernel::setEnabled();

int returnState = userMain(argc, argv);

Kernel::setDisabled();

cout<<"\nDone!";

cin.get();

return returnState;

}

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          pcb.cpp

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Kernel implementation of a user's thread

//===========================================================================//

#include "hwdepend.h"

#include "kernel.h"

#include "pcb.h"

#include "queue.h"

PCB::PCB(StackSize stackSize, Time timeSlice, Thread* thread) {

lock();

tslice = timeSlice;

sleepTime = 0;

myT = thread;

state = NEW;

forDealloc = new byte[stackSize];

currStack = initStack(&Kernel::wrapper, stackSize, forDealloc);

blocked = new Queue();

unlock();

}

PCB::~PCB() {

lock();

delete forDealloc;

delete blocked;

unlock();

}

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          queue.cpp

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   The implementation of the waiting queue

//===========================================================================//

#include <_null.h>

#include "hwdepend.h"

#include "queue.h"

void dispatch();

Elem::Elem(PCB* pcb): info(pcb), next(NULL) {}

Queue::Queue() {

lock();

first = last = NULL;

cnt =0;

unlock();

}

Queue::~Queue() {               //Queue must not be destroyed

lock();                     //if there are threads waiting in it

while(cnt) dispatch();      //because the only reference to them

unlock();                   //would be lost

}

void Queue::put(PCB* pcb) {

lock();

cnt++;

if(last==NULL) {

last = new Elem(pcb);

first = last;

unlock();

return;

}

last->next = new Elem(pcb);

last = last->next;

unlock();

}

PCB* Queue::get() {

lock();

cnt--;

if(first==NULL) {

unlock();

return NULL;

}

PCB* ret = first->info;

Elem* old = first;

first = first->next;

delete old;

if(first==NULL) last = NULL;

unlock();

return ret;

}

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          semaphor.cpp

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   User's semaphore

//===========================================================================//

#include "hwdepend.h"

#include "krnlsem.h"

#include "semaphor.h"

Semaphore::Semaphore(int init) {

lock();

myImpl = new KernelSem(init);

unlock();

}

Semaphore::~Semaphore() {

lock();

delete myImpl;

unlock();

}

void Semaphore::wait() {

lock();

myImpl->wait();

unlock();

}

void Semaphore::signal() {

lock();

myImpl->signal();

unlock();

}

int Semaphore::val() const { return myImpl->value; }

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          sleeping.cpp

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   Sleeping threads' queue implementation

//===========================================================================//

#include <_null.h>

#include "hwdepend.h"

#include "pcb.h"

#include "queue.h"

#include "schedul.h"

#include "sleeping.h"

Queue SleepingQueue::sleeping;

void SleepingQueue::put(PCB* pcb) {

lock();

sleeping.put(pcb);

unlock();

}

int SleepingQueue::tick() {

int awake = 0;

Elem* curr = sleeping.first, *prev = NULL;

while(curr!=NULL) {

curr->info->sleepTime -= TIMER_FREQ;

if(curr->info->sleepTime <= 0) {

curr->info->state = PCB::READY;

Scheduler::put(curr->info);

awake++;

if(prev!=NULL) prev->next = curr->next;

else sleeping.first = sleeping.first->next;

Elem* old = curr;

curr = curr->next;

delete old;

if(curr==NULL) sleeping.last = prev;

sleeping.cnt--;

}

else {

prev = curr;

curr = curr->next;

}

}

return awake;

}

//===========================================================================//

//   Project:       Projekat iz Operativnih sistema 1

//   File:          thread.cpp

//   Date:          August 2005

//   Author:        Darko Petrovic

//   Description:   User's thread

//===========================================================================//

#include "hwdepend.h"

#include "kernel.h"

#include "pcb.h"

#include "queue.h"

#include "schedul.h"

#include "sleeping.h"

#include "thread.h"

Time minTimeSlice() { return TIMER_FREQ; }

void dispatch() {

lock();

if(!Kernel::enabled) { unlock(); return; }

Kernel::explicitDispatch = 1;

Kernel::timerISR();

unlock();

}

Thread::Thread (StackSize stackSize, Time timeSlice) {

lock();

myPCB = new PCB(stackSize, timeSlice, this);

unlock();

}

Thread::~Thread() {

lock();

waitToComplete();

delete myPCB;

unlock();

}

void Thread::start() {

lock();

myPCB->state = PCB::READY;

Kernel::signUp();

Scheduler::put(myPCB);

unlock();

}

void Thread::waitToComplete() {

lock();

if(!Kernel::enabled) { unlock(); return; }

if(myPCB->state==PCB::NEW) { unlock(); return; }

if(myPCB->state==PCB::DONE) { unlock(); return; }

if(myPCB==(PCB*)Kernel::running) { unlock(); return; }

Kernel::running->state = PCB::BLOCKED;

myPCB->blocked->put((PCB*)Kernel::running);

dispatch();

unlock();

}

void Thread::sleep(Time t) {

lock();

if(t==0) { unlock(); return; }

myPCB->state = PCB::SLEEPING;

myPCB->sleepTime = t;

SleepingQueue::put(myPCB);

if(myPCB==(PCB*)Kernel::running) dispatch();

unlock();

}